# Concrete Columns and Walls Construction ITP Template

Reusable editable baseline for cast-in-place reinforced concrete building columns and structural walls. Covers interface/starter verification, reinforcement and embeds, formwork/alignment, pre-pour review, supply and specified tests, vertical placement/consolidation, joints, curing/stripping, geometry, defects and records. Excludes precast, slipform/jumpform specialist design, post-tensioning, retaining/tank walls, shotcrete and architectural exposed-concrete mockups unless project-specific scope is added. No formwork engineering, concrete lift limits, tolerances, strength criteria, stripping time or release authority is supplied.

| Project | [Enter project / state / parties] |
| --- | --- |
| Package | concrete-columns-walls |
| Document / revision | [Enter controlled number and revision] |
| Issue purpose | For review - proposed editable baseline |
| Boundary | Complete against project documents; remove out-of-scope activities. |

# Read before use

This is a reusable proposed baseline, not an approved project plan or completed inspection record.

Enter actual acceptance criteria, source revisions/locators, lot basis, frequency, roles and contractual control points from the project. No numeric limit, test result, qualification, signature or client release has been supplied.

Confirm scope and remove or mark conditional rows that do not apply. Resolve missing or conflicting decisions with the assigned project authority.

Complete the matching blank Record Prompts only when actual work occurs; retain source certificates and original specialist reports.

# Project inputs to resolve

Element / pour / lot: [Enter project source / decision] Identify the physical element, drawing grid/level, pour sequence and lot boundary.

Approved drawings and revisions: [Enter project source / decision] List current structural drawings, details, schedules and approved design changes relevant to this element.

Project specification and adopted standards: [Enter project source / decision] Enter exact contract clauses, adopted standards/editions and project criteria; do not infer adoption from a published edition.

Approved concrete mix and supplier submission: [Enter project source / decision] Identify approved mix, supplier, delivery requirements and any project-authorised adjustments.

Project test plan and acceptance: [Enter project source / decision] Identify specified test methods, sample/lot coverage, acceptance and designated tester/reviewer from controlled project documents.

Performing, reviewing and release roles: [Enter project source / decision] Identify actual contractor, verifier/designer, notice duties, contractual Hold/Witness points and release authority.

Project evidence and closeout requirements: [Enter project source / decision] Identify required pour records, test reports, survey, defect disposition and handover register.

Column / wall schedule and pour boundaries: [Enter project source / decision] Identify element IDs, grids, storeys/lifts, pour limits and wall/column interfaces.

Formwork and temporary-works design/release: [Enter project source / decision] Identify approved formwork design, ties, bracing, pour method, monitoring and assigned release roles.

Construction-joint and surface requirements: [Enter project source / decision] Identify approved joint locations/treatment, finish class, openings, embeds and surface-repair procedure.

Stripping / loading release requirements: [Enter project source / decision] Identify design- or project-controlled criteria, authorization and dependent-work release; no value is presumed.

# Inspection and Test Plan

Proposed baseline. Replace prompts with controlled project criteria and references before project review.

## CWL-01 · Element scope and pour-lift register

| Inspection / test | Confirm each column/wall element, grid, level, pour height/sequence, drawings and interface boundaries are defined. |
| --- | --- |
| Method | Reconcile element schedule, structural drawings, slab/footing starters, floor penetrations, wall openings and pour plan; identify adjacent trade and design interfaces. |
| Acceptance condition | For [Enter Element / pour / lot], IDs and limits agree with [Enter Column / wall schedule and pour boundaries] and current [Enter Approved drawings and revisions]; unresolved geometry, sequence or interface conflicts stay open. |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | Proposed: Before reinforcement/formwork and whenever the approved sequence or drawings change. |
| Responsibility | Proposed: [Enter Person/role performing this check] performs and records; [Enter Person/role reviewing this check] reviews only where assigned by project documents. |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Column and wall element / lift register |
| Applicability / remarks | Proposed baseline. Confirm applicability and replace with controlled project requirement. |

## CWL-02 · Starter bars, couplers and construction joint

| Inspection / test | Check the element interface with the prior pour: starter/coupler identity, exposed reinforcement, approved joint location and preparation requirements. |
| --- | --- |
| Method | Trace bars/couplers and joint detail to approved drawings; record surface preparation evidence and unresolved damage or contamination before closing forms. |
| Acceptance condition | Interface matches [Enter Approved drawings and revisions] and [Enter Construction-joint and surface requirements]; coupler requirements, joint preparation and any repair are evidenced by approved documents and assigned disposition. |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | Proposed: Before closing the next pour formwork or concealing the connection. |
| Responsibility | Proposed: [Enter Person/role performing this check] performs and records; [Enter Person/role reviewing this check] reviews only where assigned by project documents. |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Vertical element interface and joint record |
| Applicability / remarks | Proposed baseline. Confirm applicability and replace with controlled project requirement. |

## CWL-03 · Reinforcement cage, confinement and cover

| Inspection / test | Check bar marks, cage arrangement, ties/fitments, laps/couplers, spacers, specified cover and congestion around openings and embeds. |
| --- | --- |
| Method | Compare actual cage configuration with current schedules/details; record measured checks by method and locations; coordinate penetrations and cast-ins before form closure. |
| Acceptance condition | Cage and specified cover conform to [Enter Approved drawings and revisions] and [Enter Project specification and adopted standards] at documented check points; no dimensions, spacing or cover values are supplied by the baseline. |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | Proposed: Before form closure for each defined element / pour, with project coverage specified. |
| Responsibility | Proposed: [Enter Person/role performing this check] performs and records; [Enter Person/role reviewing this check] reviews only where assigned by project documents. |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Column and wall reinforcement inspection |
| Applicability / remarks | Proposed baseline. Confirm applicability and replace with controlled project requirement. |

## CWL-04 · Embedded items, openings and service coordination

| Inspection / test | Verify cast-in plates, anchors, sleeves, conduits, waterstops, blockouts and openings required by approved coordinated drawings. |
| --- | --- |
| Method | Check identifiers, locations and orientation against structural and coordinated services drawings; trace any late change to written design approval. |
| Acceptance condition | Cast-in items and openings match approved [Enter Approved drawings and revisions]; conflicts or undocumented penetrations remain open for design resolution before pour. |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | Proposed: Before form closure and after any agreed coordination change. |
| Responsibility | Proposed: [Enter Person/role performing this check] performs and records; [Enter Person/role reviewing this check] reviews only where assigned by project documents. |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Vertical element cast-in and opening register |
| Applicability / remarks | Proposed baseline. Confirm applicability and replace with controlled project requirement. |

## CWL-05 · Formwork geometry and temporary works readiness

| Inspection / test | Record dimensions, plumb/alignment, joint sealing, cleanout/access, ties and the approved formwork/falsework release for the planned pour. |
| --- | --- |
| Method | Inspect accessible formwork and compare measurements, bracing/tie arrangement and pour method to approved design and erection documents; do not independently design or certify formwork. |
| Acceptance condition | Formwork and planned placement meet [Enter Formwork and temporary-works design/release] and [Enter Approved drawings and revisions]; design adequacy and release are attributed to the assigned competent authority. |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | Proposed: Before pour for each element or approved formwork system lot; recheck after changes or disturbance. |
| Responsibility | Proposed: [Enter Person/role performing this check] performs and records; [Enter Person/role reviewing this check] reviews only where assigned by project documents. |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Column and wall formwork inspection |
| Applicability / remarks | Proposed baseline. Confirm applicability and replace with controlled project requirement. |

## CWL-06 · Pre-pour review and notice / release

| Inspection / test | Document final access, cage, joints, embeds, formwork, delivery/test arrangements and actual pre-pour decisions. |
| --- | --- |
| Method | Walk the defined element and compare records to the current ITP, pour sequence and project notices; record each party and authority separately. |
| Acceptance condition | Pre-pour status for [Enter Element / pour / lot] is traceable to [Enter Project test plan and acceptance], [Enter Formwork and temporary-works design/release] and assigned [Enter Notice and release authority]; a contractor check does not imply a third-party hold release. |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | Proposed: Each defined element / pour before discharge, subject to project notice and coverage. |
| Responsibility | Proposed: [Enter Person/role performing this check] performs and records; [Enter Person/role reviewing this check] reviews only where assigned by project documents. |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Column and wall pre-pour release register |
| Applicability / remarks | Proposed baseline. Confirm applicability and replace with controlled project requirement. |

## CWL-07 · Concrete delivery and specified fresh tests

| Inspection / test | Confirm delivered mix identity and perform only specified fresh testing/sampling with batch-to-element traceability. |
| --- | --- |
| Method | Review dockets and approved mix; record any authorized adjustment; link sample, location, test method, result, tester and lab reference to the actual element/pour. |
| Acceptance condition | Delivery and tests meet [Enter Approved concrete mix and supplier submission] and [Enter Project test plan and acceptance]; no slump, temperature, sampling rate, strength or rejection value is implied. |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | Proposed: For project-defined deliveries and test samples; use project lot/frequency rules. |
| Responsibility | Proposed: [Enter Person/role performing this check] performs and records; [Enter Person/role reviewing this check] reviews only where assigned by project documents. |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Vertical element delivery and fresh test register |
| Applicability / remarks | Proposed baseline. Confirm applicability and replace with controlled project requirement. |

## CWL-08 · Vertical placement, lift sequence and consolidation

| Inspection / test | Observe concrete discharge, vertical placement sequence, flow around cage/openings and consolidation against the approved pour method. |
| --- | --- |
| Method | Record time and element zones; compare equipment and planned lifts to controlled method; note interruptions, leakage, segregation, cage movement or access constraints. |
| Acceptance condition | Placement follows [Enter Project specification and adopted standards] and [Enter Formwork and temporary-works design/release]; actual lift limits and rates come from approved project/design documents and are not supplied here. |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | Proposed: During placement of each element using project-defined observation intervals / coverage. |
| Responsibility | Proposed: [Enter Person/role performing this check] performs and records; [Enter Person/role reviewing this check] reviews only where assigned by project documents. |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Column and wall placement log |
| Applicability / remarks | Proposed baseline. Confirm applicability and replace with controlled project requirement. |

## CWL-09 · Construction joints and continuation between lifts

| Inspection / test | Record planned or actual construction joints, interruptions, surface condition and approved continuation method between pours/lifts. |
| --- | --- |
| Method | Map joint location and time to element record; compare preparation and interface system with approved structural detail and method; refer unplanned joints. |
| Acceptance condition | Joint location, preparation and continuation comply with [Enter Construction-joint and surface requirements] and written project/design disposition; no default joint location or acceptance is given. |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | Proposed: Each planned joint and any interruption before the next pour proceeds. |
| Responsibility | Proposed: [Enter Person/role performing this check] performs and records; [Enter Person/role reviewing this check] reviews only where assigned by project documents. |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Vertical element construction joint register |
| Applicability / remarks | Proposed baseline. Confirm applicability and replace with controlled project requirement. |

## CWL-10 · Curing, stripping and geometry / defect closeout

| Inspection / test | Track specified curing and protect elements; record authorized stripping, survey/alignment, visible defects, repairs and downstream release. |
| --- | --- |
| Method | Compare actual curing and strip timing to approved project instructions; survey finished geometry with specified method; map defects and link engineer-approved repair/recheck evidence. |
| Acceptance condition | Curing, removal/loading and measured geometry follow [Enter Project specification and adopted standards] and [Enter Formwork and temporary-works design/release]; no stripping time, tolerance, repair method or release is presumed; compile [Enter Project evidence and closeout requirements]. |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | Proposed: From placement through specified curing and release milestones, then at element closeout. |
| Responsibility | Proposed: [Enter Person/role performing this check] performs and records; [Enter Person/role reviewing this check] reviews only where assigned by project documents. |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Column and wall curing, survey and defect register |
| Applicability / remarks | Proposed baseline. Confirm applicability and replace with controlled project requirement. |

# Blank matching record prompts

Complete the actual date, observation, result and responsible roles during the work. Blank cells are intentional; this template contains no completed inspection or approval.

## CWL-01 · Element scope and pour-lift register

| Field | Actual entry / evidence |
| --- | --- |
| Column / wall ID |  |
| Grid / storey / level |  |
| Pour / lift boundary |  |
| Drawing / revision |  |
| Interface / RFI ref |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

## CWL-02 · Starter bars, couplers and construction joint

| Field | Actual entry / evidence |
| --- | --- |
| Element / joint ID |  |
| Starter / coupler mark |  |
| Previous pour / record ref |  |
| Joint preparation observation |  |
| Damage / disposition ref |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

## CWL-03 · Reinforcement cage, confinement and cover

| Field | Actual entry / evidence |
| --- | --- |
| Element / grid / level |  |
| Bar / fitment mark |  |
| Lap / coupler ref |  |
| Measured cover / method |  |
| Opening / embed / congestion note |  |
| Deviation / disposition ref |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

## CWL-04 · Embedded items, openings and service coordination

| Field | Actual entry / evidence |
| --- | --- |
| Element / level |  |
| Embed / opening ID |  |
| Measured location / orientation |  |
| Coordinated drawing / revision |  |
| Change / approval ref |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

## CWL-05 · Formwork geometry and temporary works readiness

| Field | Actual entry / evidence |
| --- | --- |
| Element / form ID |  |
| Measured dimension / plumb |  |
| Measure method / equipment |  |
| Formwork design / revision |  |
| Designer / verifier / release ref |  |
| Defect / action ref |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

## CWL-06 · Pre-pour review and notice / release

| Field | Actual entry / evidence |
| --- | --- |
| Element / pour ID |  |
| Open issue / disposition |  |
| Installer / role / date |  |
| Verifier / role / date |  |
| Authority / decision / date / reference |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

## CWL-07 · Concrete delivery and specified fresh tests

| Field | Actual entry / evidence |
| --- | --- |
| Element / pour |  |
| Batch / docket |  |
| Mix ID |  |
| Sample / location |  |
| Method / equipment |  |
| Actual result / units |  |
| Tester / report ref |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

## CWL-08 · Vertical placement, lift sequence and consolidation

| Field | Actual entry / evidence |
| --- | --- |
| Element / lift |  |
| Placement time |  |
| Zone / sequence |  |
| Placement / consolidation method |  |
| Movement / leakage / interruption |  |
| Action / disposition ref |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

## CWL-09 · Construction joints and continuation between lifts

| Field | Actual entry / evidence |
| --- | --- |
| Element / joint ID |  |
| Joint / lift location |  |
| Last / next placement time |  |
| Preparation / method ref |  |
| Design / project disposition ref |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

## CWL-10 · Curing, stripping and geometry / defect closeout

| Field | Actual entry / evidence |
| --- | --- |
| Element / level |  |
| Method / product / conditions |  |
| Start / end date and times |  |
| Strip authorization / reference |  |
| Survey / dimensions / method |  |
| Defect / repair / recheck ref |  |
| Closeout dossier ref |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

# Topic guidance sources and limitations

## NSW Government Quality Management Guidelines for Construction, Appendix D (ITP guidance)

Public webpage description and Appendix D download listing; Appendix D DOCX content was not inspected.

https://www.info.buy.nsw.gov.au/resources/construction/quality-management-guidelines-for-construction

Limited methodology context only: the NSW public page identifies a construction ITP guideline/checklist resource and contract-level quality planning. The linked Appendix D document itself was not inspected, so no detailed template fields or technical acceptance rules are attributed to it.

## Queensland TMR MRTS70 Concrete (July 2022)

MRTS70 July 2022: Sections 5 (quality system), 6 (concrete class), 7 (materials), 8 (storage), 9 (mix design/acceptance), 10 (batching/mixing/transport), 11 (plastic concrete acceptance), 12 (hardened concrete acceptance incl. sampling), 13 (environmental limits), 14 (defects/rectification), and 15 (in-situ concrete: tests, falsework/formwork, placement/compaction, tolerances, stripping/loading). Section 17 covers normal-class concrete-specific requirements including placement, finishing, curing and early loading. Local searchable extract: industry-library/text/tmr-mrts70.txt.

https://www.tmr.qld.gov.au/_/media/busind/techstdpubs/specifications-and-drawings/specifications/2-bridges-and-structures/mrts70.pdf?extension=pdf&hash=BD4A48371C386F32622F6862572AA56A&rev=24517261eac44ddb86310c7980172050&sc_lang=en&size=1730809

Inspected topic-specific technical example for Queensland transport/infrastructure concrete. In-situ topics are supported by Section 15; Section 17 is only relevant to its stated normal-class scope. Use numerical or procedural criteria only where the customer project adopts this edition and relevant annexures; not a general building requirement.

## Hutchinson Builders, Construction ITPs and Reports suite (downloadable Word examples)

Groundworks ITP/Report, Concrete Place ITP/Report, Structural Steel ITP/Report; downloaded public DOCX files inspected 2026-10-06

https://subbies.hutchies.com.au/resources/videos/safety-and-quality-planning

Australian builder industry examples support high-level task sequencing, separate groundworks/concrete/steel process stages, records and lot/report fields only. Examples are explicitly marked EXAMPLE; they are not standards and provide no universal acceptance values or test rates. Original templates are not redistributed or copied.

## Standards Australia catalogue — AS 1379:2026, Specification and supply of concrete (metadata only)

Public catalogue listing: AS 1379:2026, Current, published 22 May 2026; metadata checked 7 October 2026. Designation, title, status and publication date only. Full standard text was not inspected.

https://store.standards.org.au/explore-standards/manufacturing?page=104

Edition-awareness pointer only; full standard text was not inspected. A new publication does not automatically amend a contract, adopted standard edition or MRTS70 July 2022. Confirm project adoption and applicable amendments before citing clauses or using acceptance values.

## Standards Australia catalogue — AS 3600:2018, Concrete structures (metadata only)

Public catalogue lists AS 3600:2018 as Pending Revision and amendments 1:2018 and 2:2021 as Current; metadata checked 7 October 2026. Full standard and amendment texts were not inspected.

https://store.standards.org.au/search?query=AS3600%3A2018%E2%80%93ConcreteStructures&status=Current_PendingRevision

Optional structural-reference pointer only; full standard and amendment texts were not inspected. Confirm applicability, project-adopted edition and amendments. Catalogue metadata does not establish technical compliance or supply project acceptance criteria.

Topic guidance is provided for orientation only. Confirm applicable jurisdictional adoption, approved design and contract requirements. No cited source substitutes for the complete controlled project documents or licensed standards.