# Selected Engineered Fill Placement and Compaction Inspection and Test Plan Template

Reusable baseline for source-controlled selected engineered fill in defined zones, including receipt, layer placement, conditioning, compaction process, specified compliance tests, rework and survey/handover. Excludes bulk cut/fill balancing, ordinary utility trench bedding/surround/backfill, pavement-specific acceptance unless included, stabilisation and ground improvement.

| Project | [Enter project / state / parties] |
| --- | --- |
| Package | fill-compaction |
| Document / revision | [Enter controlled number and revision] |
| Issue purpose | For review - proposed editable baseline |
| Boundary | Complete against project documents; remove out-of-scope activities. |

# Read before use

This is a reusable proposed baseline, not an approved project plan or completed inspection record.

Enter actual acceptance criteria, source revisions/locators, lot basis, frequency, roles and contractual control points from the project. No numeric limit, test result, qualification, signature or client release has been supplied.

Confirm scope and remove or mark conditional rows that do not apply. Resolve missing or conflicting decisions with the assigned project authority.

Complete the matching blank Record Prompts only when actual work occurs; retain source certificates and original specialist reports.

# Project inputs to resolve

Project jurisdiction / authority context: [Enter project source / decision] State or territory and any authority or contract scope that governs the work.

Approved civil / services drawings and revisions: [Enter project source / decision] Identify controlled plans, sections, set-out details, levels and applicable drawing items.

Project earthworks specification and adopted standards: [Enter project source / decision] Identify contract clauses, adopted editions, annexures and precedence.

Geotechnical design basis and disposition roles: [Enter project source / decision] Identify the report, design zones and who provides any specified founding or changed-ground disposition.

Material source, classification and use schedule: [Enter project source / decision] Identify approved source, permitted use and project material criteria; do not infer a class.

Project testing, lotting and reporting plan: [Enter project source / decision] Enter nominated methods, test locations, lot definition, rate, acceptance and laboratory role.

Survey control, geometry and closeout requirements: [Enter project source / decision] Identify control, design geometry, tolerances, as-built format and responsible survey role.

Service clearance and permit process: [Enter project source / decision] Identify current service information, locating, protection and permit requirements.

Service and work-package interfaces: [Enter project source / decision] Identify exact trench, building, pavement or service boundaries and owner handover/testing responsibilities.

Performing, reviewing and contractual release roles: [Enter project source / decision] Identify assigned roles, notice period, evidence, and contractual Hold/Witness or release conditions.

Project records and handover requirements: [Enter project source / decision] Identify required registers, reports, photos, dispositions and handover submission.

# Inspection and Test Plan

Proposed baseline. Replace prompts with controlled project criteria and references before project review.

## SEF-01 · Engineered fill scope and acceptance basis

| Inspection / test | Define each selected engineered fill zone, intended function, design elevation, required material class and acceptance evidence. |
| --- | --- |
| Method | Review approved drawings, geotechnical/design documents, earthworks specification and test/lot plan; record the governing revision and unresolved decisions. |
| Acceptance condition | Zone boundaries, use, required material, placement method, acceptance criteria, lotting and responsibility are stated in controlled project documents; no value is inferred. [Enter Approved civil / services drawings and revisions] [Enter Geotechnical design basis and disposition roles] [Enter Project earthworks specification and adopted standards] [Enter Project testing, lotting and reporting plan] |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | Before sourcing, placement or changes to zone/design basis. |
| Responsibility | Proposed: assigned earthworks supervisor / quality role performs and records; project-assigned reviewer acts only within documented authority. Confirm project roles: [Enter Performing, reviewing and contractual release roles]. |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Selected fill zone and criteria register |
| Applicability / remarks | Include only for work shown in this package scope. Proposed baseline. Confirm applicability and replace with controlled project requirement. |

## SEF-02 · Fill source and material approval

| Inspection / test | Verify source identity and project evidence for each selected engineered fill material before delivery/use. |
| --- | --- |
| Method | Compare source proposal, representative material identification and required test/certification reports with project material schedule and nominated approval route. |
| Acceptance condition | Source, material type and intended zone match the approved project record; required evidence is complete and current for the material/source presented. [Enter Material source, classification and use schedule] [Enter Project testing, lotting and reporting plan] [Enter Project earthworks specification and adopted standards] |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | Before first use of each source/material and when source or material changes. |
| Responsibility | Proposed: assigned earthworks supervisor / quality role performs and records; project-assigned reviewer acts only within documented authority. Confirm project roles: [Enter Performing, reviewing and contractual release roles]. |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Engineered fill source approval register |
| Applicability / remarks | Include only for work shown in this package scope. Proposed baseline. Confirm applicability and replace with controlled project requirement. |

## SEF-03 · Receiving formation readiness

| Inspection / test | Confirm the prepared receiving surface is identified and accepted for selected engineered fill placement. |
| --- | --- |
| Method | Inspect the surface, drainage and boundaries; compare set-out and documented geotechnical/design disposition; confirm required preceding records are available. |
| Acceptance condition | Receiving surface and any specified treatment/release match project drawings and written dispositions; unresolved water, soft areas or conflicts are referred before covering. [Enter Approved civil / services drawings and revisions] [Enter Geotechnical design basis and disposition roles] [Enter Survey control, geometry and closeout requirements] [Enter Performing, reviewing and contractual release roles] |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | Before the first fill layer in each zone and after re-preparation. |
| Responsibility | Proposed: assigned earthworks supervisor / quality role performs and records; project-assigned reviewer acts only within documented authority. Confirm project roles: [Enter Performing, reviewing and contractual release roles]. |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Receiving formation inspection |
| Applicability / remarks | Include only for work shown in this package scope. Proposed baseline. Confirm applicability and replace with controlled project requirement. |

## SEF-04 · Delivery identity and material traceability

| Inspection / test | Check each delivery or defined delivery group is linked to the approved source, material and intended zone. |
| --- | --- |
| Method | Review docket/load records and observe material identity/condition; reconcile source, material designation and destination with the approval register. |
| Acceptance condition | Delivery is identifiable and consistent with the accepted material for the intended zone; discrepancies are segregated pending project decision. [Enter Material source, classification and use schedule] [Enter Project records and handover requirements] |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | Each delivery or project-defined delivery group before placement. |
| Responsibility | Proposed: assigned earthworks supervisor / quality role performs and records; project-assigned reviewer acts only within documented authority. Confirm project roles: [Enter Performing, reviewing and contractual release roles]. |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Selected fill delivery register |
| Applicability / remarks | Include only for work shown in this package scope. Proposed baseline. Confirm applicability and replace with controlled project requirement. |

## SEF-05 · Layer area, thickness and placement sequence

| Inspection / test | Inspect the placed layer and record its area and thickness basis before conditioning/compaction. |
| --- | --- |
| Method | Observe spreading, segregation and layer boundaries; measure or obtain the project-required thickness evidence using the specified method. |
| Acceptance condition | Material is placed in the approved zone and sequence; layer dimensions and continuity meet the project-stated requirements and are recorded against the source criteria. [Enter Approved civil / services drawings and revisions] [Enter Project earthworks specification and adopted standards] [Enter Project testing, lotting and reporting plan] |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | Each layer/zone and material change before compaction. |
| Responsibility | Proposed: assigned earthworks supervisor / quality role performs and records; project-assigned reviewer acts only within documented authority. Confirm project roles: [Enter Performing, reviewing and contractual release roles]. |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Engineered fill layer placement record |
| Applicability / remarks | Include only for work shown in this package scope. Proposed baseline. Confirm applicability and replace with controlled project requirement. |

## SEF-06 · Moisture conditioning and compaction method

| Inspection / test | Verify the approved conditioning and compaction method is being followed for the identified material and layer. |
| --- | --- |
| Method | Observe conditioning and plant/method; compare with approved procedure or project trial where required; log actual observations only where specified. |
| Acceptance condition | Method and any moisture/plant constraints follow the approved project procedure; no target moisture, equipment, pass count or density is assumed by this baseline. [Enter Project earthworks specification and adopted standards] [Enter Material source, classification and use schedule] [Enter Project testing, lotting and reporting plan] |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | During each applicable layer and whenever material, weather or method changes. |
| Responsibility | Proposed: assigned earthworks supervisor / quality role performs and records; project-assigned reviewer acts only within documented authority. Confirm project roles: [Enter Performing, reviewing and contractual release roles]. |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Layer conditioning and compaction process log |
| Applicability / remarks | Include only for work shown in this package scope. Proposed baseline. Confirm applicability and replace with controlled project requirement. |

## SEF-07 · Compaction operation and layer completion

| Inspection / test | Inspect the controlled compaction process and identify completed layer boundaries for testing or next-stage work. |
| --- | --- |
| Method | Observe the approved operation across the identified area; map completed and reworked portions to the lot/layer plan without substituting visual observation for specified testing. |
| Acceptance condition | The specified method covers the recorded area and layer; any omitted, disturbed or visibly deficient area is corrected and identified for required verification before concealment. [Enter Project earthworks specification and adopted standards] [Enter Project testing, lotting and reporting plan] |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | Per defined layer/lot during placement and before the next layer obscures it. |
| Responsibility | Proposed: assigned earthworks supervisor / quality role performs and records; project-assigned reviewer acts only within documented authority. Confirm project roles: [Enter Performing, reviewing and contractual release roles]. |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Compaction activity and layer map |
| Applicability / remarks | Include only for work shown in this package scope. Proposed baseline. Confirm applicability and replace with controlled project requirement. |

## SEF-08 · Specified compliance tests and result disposition

| Inspection / test | Track project-required samples/tests by material, layer, lot, location, method and report; manage failures to written disposition. |
| --- | --- |
| Method | Check test request and report traceability against project test plan; compare reported results only with the cited project acceptance clause and identify retest/rework link. |
| Acceptance condition | Required tests and reports cover the defined lot under project criteria; results are assessed by the nominated method; failed or missing results remain open until documented disposition and verification. [Enter Project testing, lotting and reporting plan] [Enter Project earthworks specification and adopted standards] [Enter Performing, reviewing and contractual release roles] |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | At the project-defined lot/sample trigger and before dependent work; exact rate and method are project inputs. |
| Responsibility | Proposed: assigned earthworks supervisor / quality role performs and records; project-assigned reviewer acts only within documented authority. Confirm project roles: [Enter Performing, reviewing and contractual release roles]. |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Selected fill test and disposition register |
| Applicability / remarks | Include only for work shown in this package scope. Proposed baseline. Confirm applicability and replace with controlled project requirement. |

## SEF-09 · Finished zone survey, protection and handover

| Inspection / test | Verify completed zone geometry, test dossier, protection condition and receiving-trade interface. |
| --- | --- |
| Method | Compare survey and test register with project closeout requirements; walk the zone for damage, drainage or disturbance after test/acceptance. |
| Acceptance condition | Specified geometry and records are submitted against current project criteria; damage or outstanding results are resolved or carried as a documented hold before handover. [Enter Survey control, geometry and closeout requirements] [Enter Project testing, lotting and reporting plan] [Enter Project records and handover requirements] |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | At completion of each zone and immediately before handover or concealment. |
| Responsibility | Proposed: assigned earthworks supervisor / quality role performs and records; project-assigned reviewer acts only within documented authority. Confirm project roles: [Enter Performing, reviewing and contractual release roles]. |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Selected fill completion and handover index |
| Applicability / remarks | Include only for work shown in this package scope. Proposed baseline. Confirm applicability and replace with controlled project requirement. |

# Blank matching record prompts

Complete the actual date, observation, result and responsible roles during the work. Blank cells are intentional; this template contains no completed inspection or approval.

## SEF-01 · Engineered fill scope and acceptance basis

| Field | Actual entry / evidence |
| --- | --- |
| Zone / layer purpose |  |
| Design / geotechnical ref. |  |
| Material / acceptance source locator |  |
| Lot / test plan ref. |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

## SEF-02 · Fill source and material approval

| Field | Actual entry / evidence |
| --- | --- |
| Quarry / source / supplier |  |
| Material ID / description |  |
| Test / certificate ref. |  |
| Approved use zone |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

## SEF-03 · Receiving formation readiness

| Field | Actual entry / evidence |
| --- | --- |
| Zone / area |  |
| Observed level / drawing locator |  |
| Surface / moisture / drainage observation |  |
| Disposition / release ref. |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

## SEF-04 · Delivery identity and material traceability

| Field | Actual entry / evidence |
| --- | --- |
| Docket / load ID |  |
| Approved source ID |  |
| Material ID / description |  |
| Zone / layer reference |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

## SEF-05 · Layer area, thickness and placement sequence

| Field | Actual entry / evidence |
| --- | --- |
| Layer ID / elevation |  |
| Area / lot boundary |  |
| Observed layer thickness / unit |  |
| Material / delivery references |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

## SEF-06 · Moisture conditioning and compaction method

| Field | Actual entry / evidence |
| --- | --- |
| Layer / lot ID |  |
| Plant / method reference |  |
| Observed conditioning / actual reading if required |  |
| Approved method / trial ref. |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

## SEF-07 · Compaction operation and layer completion

| Field | Actual entry / evidence |
| --- | --- |
| Lot / boundary plan ref. |  |
| Layer ID / area |  |
| Method / plant reference |  |
| Rework area / instruction ref. |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

## SEF-08 · Specified compliance tests and result disposition

| Field | Actual entry / evidence |
| --- | --- |
| Test / method / date |  |
| Lot / layer / location |  |
| Actual result / unit |  |
| Project clause / disposition / retest ref. |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

## SEF-09 · Finished zone survey, protection and handover

| Field | Actual entry / evidence |
| --- | --- |
| Zone / design reference |  |
| As-built survey / revision |  |
| Test register / outstanding items |  |
| Receiving party / handover ref. |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

# Topic guidance sources and limitations

## Queensland Department of Transport and Main Roads, MRTS04 General Earthworks (March 2025)

Inspected official PDF and local searchable extract industry-library/text/tmr-mrts04.txt. Topic locators: 1.1-1.2 scope (printed specification pp. 1-2); 5.2-5.6 procedures, testing/lotting (printed pp. 15-19); 6 geometry (printed pp. 19-20); 8 topsoil (printed pp. 22-24); 9 unsuitable material (printed pp. 25-26); 13 excavation (printed pp. 34-38); 14 Earth Fill (printed pp. 38-47); 15 Compaction (printed pp. 48-53); 19 Backfill (printed pp. 67-74).

https://www.tmr.qld.gov.au/-/media/busind/techstdpubs/Specifications-and-drawings/Specifications/3-Roadworks-Drainage-Culverts-and-Geotechnical/MRTS04.pdf

Inspected topic example for Queensland road and bridge earthworks only. Supports identification of process topics, documentary controls, materials, geometry, excavation, fill, compaction, backfill and the fact that lot/test rules are specified. Its technical values, tests, frequencies, lot sizes, control points and roles are not defaults; use only if the project contract adopts this edition and applicable annexures.

## Hutchinson Builders, Safety & Quality Planning: 010 Groundworks ITP and Report (EXAMPLE)

Official downloads page lists 010 Groundworks ITP and 010 Groundworks Report; both DOCX files downloaded and text-extracted 2026-10-07 from the linked Toolbox files. ITP table rows 3-11 cover bulk set-out, fill source, excavation, fill operations, bottom/surface preparation, compaction/soil-strength records and final inspection; rows 12-25 cover detailed earthworks, trench/pit bottoms, bedding/blinding, backfill, regulatory records and closeout. Matching report table rows 2-4 capture lot/location/open-close/comments/signature; rows 8-32 carry process, specification, acceptance, pass/fail, CAR, initials and date.

https://subbies.hutchies.com.au/resources/videos/safety-and-quality-planning

Builder example marked EXAMPLE; used only to check broad staging and useful lot/report fields. Many acceptance/frequency/role cells are blank. It does not set universal criteria, regulatory steps or release authority. Original DOCX files are not redistributed.

## Construction Quality Australia, How to create an Inspection and Test Plan

Official industry guide, sections 1-4, previously inspected and summarized in skills/au-construction-itp/references/template-writing-guide.md.

https://www.cqa.org.au/post/guide-how-to-create-an-inspection-and-test-plan-itp

Method structure only: sequence checkable activities and specify method, acceptance source, trigger/frequency, responsibility, control and records. Does not substantiate earthworks technical limits or project requirements.

Topic guidance is provided for orientation only. Confirm applicable jurisdictional adoption, approved design and contract requirements. No cited source substitutes for the complete controlled project documents or licensed standards.