# Reinforcement Inspection Checklist

Blank project field record  |  Complete from current approved documents and site observations

Scope

Included: Steel reinforcement installed in cast-in-place reinforced slabs, pad footings and strip footings, from document/bar-mark check through pre-concealment inspection.

Excluded: Precast manufacture, post-tensioning/prestressing, piles, shotcrete, bridge/highway special provisions, welding/NDT acceptance, structural design/rebar design verification, concrete delivery/testing/placement, and whole-pour release.

Use and project setup

Project field: Project / contractor  ______________________________________________

Project field: Element / grid / inspection zone  ______________________________________________

Project field: Pour or lot reference  ______________________________________________

Project field: Inspection date  ______________________________________________

Project field: Structural drawing / revision  ______________________________________________

Project field: Bar schedule / revision  ______________________________________________

Before inspection: Element, grid and pour/lot IDs — Defines the physical steel package inspected and links results to the pour record.

Before inspection: Current structural drawings and bar schedule, including revisions — Provides actual marks, layout, details and dimensions.

Before inspection: Project reinforcement specification and adopted standard edition — Supplies contractual product, tolerance, records and installation criteria.

Before inspection: Approved steel/coupler submittals and delivery traceability records — Links installed marks/products to evidence.

Before inspection: Project inspection, substitution and NCR procedures — Defines assigned roles, control points and closeout process.

How to use

1. Confirm physical scope, controlled documents, measurement method, required records and assigned roles.

2. Inspect a defined zone/lot; cite criterion and actual document revision for each check.

3. Record actual measurements only where required; attach traceability and photo evidence by reference.

4. For deviations, identify affected steel, prevent concealment where required, obtain written disposition and record recheck.

5. Keep field observations separate from designer approval and project release.

Important: Blank field checklist for project adaptation. Use actual approved documents and project acceptance criteria. Record only observations made. This is not design approval, a temporary-works certificate, contractual Hold/Witness Point release, or permission to place, strip or load.

Inspection record

Project: _____________________  Location / room / element: _____________________  Lot / area: _____________________

RE-01 Inspection package and reinforcement documents

| What to observe | Identify element, pour/lot boundaries, current structural drawings, bar schedule, revisions and project inspection/check requirements. |
| --- | --- |
| How to check | Review the controlled document register against the element and schedule marks at the workface; note superseded or conflicting copies. |
| Acceptance basis | Use current approved project drawings, bar schedule, specification and ITP; record unresolved conflict rather than choosing a revision. |
| Reference to verify | Controlled document register, approved structural drawings, bar schedule, reinforcement specification and project ITP. |
| Project criterion / actual approved document and revision | ______________________________________________________________ |

| Record field | Actual value / reference |
| --- | --- |
| element / grid |  |
| drawing / rev |  |
| bar schedule / rev |  |
| open item ref |  |
| Result / status (Pass / Fail / N/A / Pending) |  |
| Inspector name / role / date |  |
| Evidence / photo / report ref |  |
| Issue / disposition / recheck evidence and date |  |

Inspection record

Project: _____________________  Location / room / element: _____________________  Lot / area: _____________________

RE-02 Delivery identity and traceability

| What to observe | Inspect labels/tags, bundle or mesh marks and delivery documents for identity to scheduled bar marks, product description and source traceability required by the job. |
| --- | --- |
| How to check | Compare delivered bundles/mesh with schedule and dockets/certificates; link identity to storage location and element allocation. |
| Acceptance basis | Apply only project procurement/specification traceability requirements and scheduled marks; do not impose TMR registration unless adopted by contract. |
| Reference to verify | Approved bar schedule, material submittal, delivery docket and project traceability requirements. |
| Project criterion / actual approved document and revision | ______________________________________________________________ |

| Record field | Actual value / reference |
| --- | --- |
| delivery docket |  |
| bundle / bar mark |  |
| certificate ref |  |
| allocated location |  |
| Result / status (Pass / Fail / N/A / Pending) |  |
| Inspector name / role / date |  |
| Evidence / photo / report ref |  |
| Issue / disposition / recheck evidence and date |  |

Inspection record

Project: _____________________  Location / room / element: _____________________  Lot / area: _____________________

RE-03 Storage and visible steel condition

| What to observe | Observe stacking/separation, ground contact, physical damage, bends/kinks and visible contamination relevant to the specified product. |
| --- | --- |
| How to check | Walk stock before fixing; inspect representative bundles and suspect bars; photograph defects and identify affected marks. |
| Acceptance basis | Compare with project specification and approved material submittal; seek direction where condition may affect specified use. |
| Reference to verify | Project reinforcement specification, approved material submittal and written condition disposition. |
| Project criterion / actual approved document and revision | ______________________________________________________________ |

| Record field | Actual value / reference |
| --- | --- |
| bar mark / bundle |  |
| storage location |  |
| condition / photo ref |  |
| disposition ref |  |
| Result / status (Pass / Fail / N/A / Pending) |  |
| Inspector name / role / date |  |
| Evidence / photo / report ref |  |
| Issue / disposition / recheck evidence and date |  |

Inspection record

Project: _____________________  Location / room / element: _____________________  Lot / area: _____________________

RE-04 Cut, bend and bar-mark conformity

| What to observe | Compare installed cut/bent shapes, schedule marks and orientation with approved bending/bar schedule, including any project-controlled site re-bending. |
| --- | --- |
| How to check | Trace selected bars to bending schedule and inspect shape/orientation; review authorization and method for permitted field adjustment. |
| Acceptance basis | Use current approved schedule and project-approved fabrication/adjustment procedure; no generic bend radius, length or tolerance is supplied. |
| Reference to verify | Approved bar bending schedule and project-approved fabrication or field-adjustment procedure. |
| Project criterion / actual approved document and revision | ______________________________________________________________ |

| Record field | Actual value / reference |
| --- | --- |
| bar mark / location |  |
| schedule shape / rev |  |
| observed shape |  |
| approval / disposition ref |  |
| Result / status (Pass / Fail / N/A / Pending) |  |
| Inspector name / role / date |  |
| Evidence / photo / report ref |  |
| Issue / disposition / recheck evidence and date |  |

Inspection record

Project: _____________________  Location / room / element: _____________________  Lot / area: _____________________

RE-05 Bar arrangement, spacing and layer position

| What to observe | Observe installed bar marks, direction, layer, orientation and relative spacing against structural details, including congestion zones. |
| --- | --- |
| How to check | Check identified grids/sections using project measurement method; record actual readings where required by drawings/specification. |
| Acceptance basis | Use actual structural drawing, bar schedule and project tolerance; no universal spacing or tolerance applies. |
| Reference to verify | Approved structural drawings, bar schedule and project dimensional criteria. |
| Project criterion / actual approved document and revision | ______________________________________________________________ |

| Record field | Actual value / reference |
| --- | --- |
| grid / section |  |
| bar marks / layer |  |
| criterion / drawing ref |  |
| actual measurement |  |
| Result / status (Pass / Fail / N/A / Pending) |  |
| Inspector name / role / date |  |
| Evidence / photo / report ref |  |
| Issue / disposition / recheck evidence and date |  |

Inspection record

Project: _____________________  Location / room / element: _____________________  Lot / area: _____________________

RE-06 Laps, anchorage, couplers and termination details

| What to observe | Inspect shown splice locations/types, lap/anchorage arrangement, bar ends and coupler identity/engagement evidence specified for the element. |
| --- | --- |
| How to check | Compare critical locations with structural details; check product and installation records for specified mechanical splices. |
| Acceptance basis | Follow exact project drawings, adopted specification/standard and approved coupler submittal; do not infer lap lengths, embedment or coupler acceptance class. |
| Reference to verify | Structural splice/anchorage details, adopted project specification and approved coupler instructions. |
| Project criterion / actual approved document and revision | ______________________________________________________________ |

| Record field | Actual value / reference |
| --- | --- |
| splice / location |  |
| detail / drawing ref |  |
| product / batch |  |
| installation / disposition ref |  |
| Result / status (Pass / Fail / N/A / Pending) |  |
| Inspector name / role / date |  |
| Evidence / photo / report ref |  |
| Issue / disposition / recheck evidence and date |  |

Inspection record

Project: _____________________  Location / room / element: _____________________  Lot / area: _____________________

RE-07 Supports, spacers and specified cover

| What to observe | Observe chair/spacer product and placement, support contact/stability and reinforcement position relative to formed faces or substrate. |
| --- | --- |
| How to check | Inspect support type/locations; check project-identified cover points with suitable verified measuring method when required. |
| Acceptance basis | Use project cover schedule/details, accepted products and tolerances; record actual values where project calls for measurement. |
| Reference to verify | Project cover schedule, support/spacer details and approved products. |
| Project criterion / actual approved document and revision | ______________________________________________________________ |

| Record field | Actual value / reference |
| --- | --- |
| support / location |  |
| cover criterion / ref |  |
| actual cover / unit / method |  |
| recheck evidence |  |
| Result / status (Pass / Fail / N/A / Pending) |  |
| Inspector name / role / date |  |
| Evidence / photo / report ref |  |
| Issue / disposition / recheck evidence and date |  |

Inspection record

Project: _____________________  Location / room / element: _____________________  Lot / area: _____________________

RE-08 Fixing, restraint and placement stability

| What to observe | Observe ties/fixings, cage rigidity, support stability and displacement risk from planned worker access and documented placing method. |
| --- | --- |
| How to check | Review approved placing/access method; inspect congested, edge, opening and support zones; record project-required movement checks. |
| Acceptance basis | Use project-approved fixing details and construction/placement method; do not infer universal tie frequency or load resistance. |
| Reference to verify | Approved fixing details and concrete placement/access method. |
| Project criterion / actual approved document and revision | ______________________________________________________________ |

| Record field | Actual value / reference |
| --- | --- |
| zone / bar IDs |  |
| fixing observed |  |
| placing method ref |  |
| direction / recheck ref |  |
| Result / status (Pass / Fail / N/A / Pending) |  |
| Inspector name / role / date |  |
| Evidence / photo / report ref |  |
| Issue / disposition / recheck evidence and date |  |

Inspection record

Project: _____________________  Location / room / element: _____________________  Lot / area: _____________________

RE-09 Surface, projection and reinforcement exceptions

| What to observe | Observe contamination/coating or loose material that may affect specified performance, projecting bars at interfaces, and outstanding substitutions/NCRs. |
| --- | --- |
| How to check | Walk steel zone before concealment; cross-check substitution/NCR register and prior-pour interfaces; record condition and closeout reference. |
| Acceptance basis | Use project specification and written designer/project disposition; no blanket rust rejection threshold or cleaning rule is imposed. |
| Reference to verify | Project specification, approved substitutions and written NCR/designer dispositions. |
| Project criterion / actual approved document and revision | ______________________________________________________________ |

| Record field | Actual value / reference |
| --- | --- |
| zone / interface |  |
| condition observed |  |
| NCR / substitution ref |  |
| action / closeout ref |  |
| Result / status (Pass / Fail / N/A / Pending) |  |
| Inspector name / role / date |  |
| Evidence / photo / report ref |  |
| Issue / disposition / recheck evidence and date |  |

Issues and corrective action

Project: _____________________  Location / room / element: _____________________  Inspection date: _____________________

| Item / date | Issue and immediate action | Disposition / corrective instruction | Recheck evidence / date | Responsible role / reference |
| --- | --- | --- | --- | --- |

Unresolved items: identify affected area, status, person notified, instruction / NCR reference and any condition before concealment or next work.

Reinforcement inspection handover

Project: _____________________  Location / room / element: _____________________  Date: _____________________

Inspection record status

[  ] Inspection record complete

[  ] Action / recheck outstanding

[  ] Design or project disposition required

This status records the steel inspection handover only. It does not mean ready to pour, approve reinforcement design, release a contractual control point, or authorize placement. Carry open items into the project ITP/pre-pour process.

Handover roles and references

Field inspector name / role / date  ______________________________________________

Receiving project role / date  ______________________________________________

Open NCR, RFI or disposition reference  ______________________________________________

Field inspector record

• Field inspector name/role/date  ______________________________________________

• Inspection lot, element and boundaries  ______________________________________________

• Observation and evidence references  ______________________________________________

Design check record

• Design document and revision  ______________________________________________

• Designer/checker identity and project-assigned role  ______________________________________________

• Design response/calculation/RFI reference  ______________________________________________

Project / contractual release record

• Release decision-maker role assigned by project/contract  ______________________________________________

• Decision status and conditions  ______________________________________________

• Basis: inspection/NCR references and control-point clause  ______________________________________________

• Date/time and notice/release record reference  ______________________________________________

Role boundary: Inspector records field observations; design author/checker and project release authority remain separate records/roles. This signature does not certify design, approve a substitution, release a Hold Point or authorize placement.

Signatures record the named person’s stated role and entry only. They do not by themselves approve design, certify compliance, release a contractual control point or authorize the next activity.

Source notes

These references explain topic and record design. They do not supply universal project acceptance values. Confirm the contract documents, applicable jurisdiction and approved product/system before use.

Cavoren cast-in-place concrete baseline

Role: Topic and sequence context only; not a project specification or independent technical authority.

Locator: Original proposed concrete baseline rows CON-01 to CON-08

Source URL: https://itp.cavoren.net/templates/concrete/

Queensland TMR Ancillary MRTS71 Reinforcing Steel / ATS 5310, July 2025

Role: Topic guidance from a Queensland transport/infrastructure specification only. Do not import its values, registrations, frequencies, roles or hold points into general building work.

Locator: Clauses 1.1–1.4; 4.1; 5.1–5.19; 6.1–6.9; 7.1–7.8; 8.1–8.5; 9.1–9.9; Sections 10–12; Appendix A. TMR listing says ancillary harmonised specifications are information only and not for use in contracts.

Source URL: https://www.tmr.qld.gov.au/_/media/busind/techstdpubs/specifications-and-drawings/specifications/2-bridges-and-structures/mrts71.pdf?extension=pdf&hash=436D56549EF2693B1E5CC9DAD5DCF1D6&rev=c09d0ebf1e8149bfbab395d06076d9d1&sc_lang=en&size=695385

Queensland TMR MRTS70 Concrete, July 2022

Role: Topic guidance only for concrete/formwork interface. Queensland transport/infrastructure contract criteria are not universal building requirements.

Locator: Sections 15.5–15.9 (formwork/falsework, reinforcement, placement/compaction, early loading).

Source URL: https://www.tmr.qld.gov.au/_/media/busind/techstdpubs/specifications-and-drawings/specifications/2-bridges-and-structures/mrts70.pdf?extension=pdf&hash=BD4A48371C386F32622F6862572AA56A&rev=24517261eac44ddb86310c7980172050&sc_lang=en&size=1730809