# Free Tiling Work Package Inspection and Test Plan Template

Ordinary internal ceramic/porcelain wall and floor tiling within a building, including substrate/readiness handoff, set-out, adhesive fixing, grout, movement/junction interfaces and closeout. Waterproofing design/installation/certification, stone, external facades, pools, structural substrate repair, wet-area compliance pathway selection and project-specific numerical tolerances are excluded or treated as interfaces.

| Project | [Enter project / state / parties] |
| --- | --- |
| Package | tiling |
| Document / revision | [Enter controlled number and revision] |
| Issue purpose | For review - proposed editable baseline |
| Boundary | Complete against project documents; remove out-of-scope activities. |

# Read before use

This is a reusable proposed baseline, not an approved project plan or completed inspection record.

Enter actual acceptance criteria, source revisions/locators, lot basis, frequency, roles and contractual control points from the project. No numeric limit, test result, qualification, signature or client release has been supplied.

Confirm scope and remove or mark conditional rows that do not apply. Resolve missing or conflicting decisions with the assigned project authority.

Complete the matching blank Record Prompts only when actual work occurs; retain source certificates and original specialist reports.

# Project inputs to resolve

Project jurisdiction and approval context: [Enter project source / decision] Identify state or territory, building class/use where relevant, authority and applicable approvals.

Approved drawings, schedules and revisions: [Enter project source / decision] Identify the exact controlled plan, detail, schedule, room/zone/line locator and revision.

Contract specification and adopted standards: [Enter project source / decision] Record the project clauses, adopted editions, annexures and precedence; verify actual adoption.

Approved materials and product system: [Enter project source / decision] Identify approved products, batch/source, technical data and installation instructions.

Approved installation method and procedures: [Enter project source / decision] Identify approved procedure, work method, manufacturer instructions and current revision.

Assigned performer, reviewer and external control roles: [Enter project source / decision] Record named roles, authority, required notices and any contract-assigned hold/witness/release.

Project records and handover submission rules: [Enter project source / decision] Identify required record formats, photographs, certificates, reports, recipients and closeout process.

Trade, authority and work-package interfaces: [Enter project source / decision] Define boundaries, predecessor readiness, penetrations/services, other trades and acceptance owner.

Project inspection/testing plan and acceptance basis: [Enter project source / decision] Copy actual method, instrument/lab, locations, lot, frequency, acceptance and report requirements.

Site safety, access and environmental controls: [Enter project source / decision] Identify site-specific risk controls, permits, access arrangements and environmental obligations.

# Inspection and Test Plan

Proposed baseline. Replace prompts with controlled project criteria and references before project review.

## TIL-01 · Scope, room schedule and tile-system release

| Inspection / test | Confirm room/zone, wall/floor surfaces, tile selection, setting-out intent, substrate type, interfaces and governing schedule are defined. |
| --- | --- |
| Method | Cross-check current room finishes schedule, tile schedule, elevations, details, approved samples and project scope; raise conflicts before work. |
| Acceptance condition | Project source and approved installation details identify the applicable scope, requirements and evidence for this location; record any gap or variance for assigned disposition. [Enter Assigned performer, reviewer and external control roles] [Enter Project inspection/testing plan and acceptance basis] [Enter Project records and handover submission rules] |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | At the work stage identified by the project inspection plan; repeat when the applicable location, installation condition or controlled source changes. [Enter Project inspection/testing plan and acceptance basis] |
| Responsibility | Proposed: assigned trade supervisor / quality role records the check; review and external sign-off remain with project-assigned persons acting within documented authority. [Enter Assigned performer, reviewer and external control roles] |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Tiling zone release register |
| Applicability / remarks | Internal building floor and wall finishes only; floor and wall are separate surfaces within one package. Proposed baseline. Confirm applicability and replace with controlled project requirement. |

## TIL-02 · Tile, adhesive, grout and accessory receipt

| Inspection / test | Verify delivered products and accessories match approved selections and are traceable to the intended zone. |
| --- | --- |
| Method | Review approvals, packaging/labels, batch references, condition and supplier documentation against schedule and product instructions. |
| Acceptance condition | Project source and approved installation details identify the applicable scope, requirements and evidence for this location; record any gap or variance for assigned disposition. [Enter Assigned performer, reviewer and external control roles] [Enter Project inspection/testing plan and acceptance basis] [Enter Project records and handover submission rules] |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | At the work stage identified by the project inspection plan; repeat when the applicable location, installation condition or controlled source changes. [Enter Project inspection/testing plan and acceptance basis] |
| Responsibility | Proposed: assigned trade supervisor / quality role records the check; review and external sign-off remain with project-assigned persons acting within documented authority. [Enter Assigned performer, reviewer and external control roles] |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Tile and setting-material receipt record |
| Applicability / remarks | Does not approve substitutions or infer compatibility. Proposed baseline. Confirm applicability and replace with controlled project requirement. |

## TIL-03 · Substrate readiness and trade handover

| Inspection / test | Inspect the released substrate for identity, condition, cleanliness, support and project-defined readiness before tiling. |
| --- | --- |
| Method | Review substrate handover and relevant drawings/product instructions; document visible defects, contamination, movement or moisture evidence only where project documents require it. |
| Acceptance condition | Project source and approved installation details identify the applicable scope, requirements and evidence for this location; record any gap or variance for assigned disposition. [Enter Assigned performer, reviewer and external control roles] [Enter Project inspection/testing plan and acceptance basis] [Enter Project records and handover submission rules] |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | At the work stage identified by the project inspection plan; repeat when the applicable location, installation condition or controlled source changes. [Enter Project inspection/testing plan and acceptance basis] |
| Responsibility | Proposed: assigned trade supervisor / quality role records the check; review and external sign-off remain with project-assigned persons acting within documented authority. [Enter Assigned performer, reviewer and external control roles] |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Substrate readiness and handover record |
| Applicability / remarks | Waterproofing installation and certification are by the separately assigned system/contract role. Proposed baseline. Confirm applicability and replace with controlled project requirement. |

## TIL-04 · Layout, datum and edge/junction set-out

| Inspection / test | Check the approved tile layout, datum, cuts, thresholds, openings and junction details are transferred to the work area. |
| --- | --- |
| Method | Compare dry layout or marked set-out with controlled elevations, tile schedule and approved samples; record requested design decisions. |
| Acceptance condition | Project source and approved installation details identify the applicable scope, requirements and evidence for this location; record any gap or variance for assigned disposition. [Enter Assigned performer, reviewer and external control roles] [Enter Project inspection/testing plan and acceptance basis] [Enter Project records and handover submission rules] |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | At the work stage identified by the project inspection plan; repeat when the applicable location, installation condition or controlled source changes. [Enter Project inspection/testing plan and acceptance basis] |
| Responsibility | Proposed: assigned trade supervisor / quality role records the check; review and external sign-off remain with project-assigned persons acting within documented authority. [Enter Assigned performer, reviewer and external control roles] |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Tile set-out check record |
| Applicability / remarks | Layout criteria and visual expectations remain project-controlled. Proposed baseline. Confirm applicability and replace with controlled project requirement. |

## TIL-05 · Tile cutting, drilling and edge preparation

| Inspection / test | Verify cuts, openings and edge treatments are coordinated with fixtures, trims and adjacent finishes. |
| --- | --- |
| Method | Inspect representative work locations against coordinated service/architectural details and tile product requirements; record chips or damage for disposition. |
| Acceptance condition | Project source and approved installation details identify the applicable scope, requirements and evidence for this location; record any gap or variance for assigned disposition. [Enter Assigned performer, reviewer and external control roles] [Enter Project inspection/testing plan and acceptance basis] [Enter Project records and handover submission rules] |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | At the work stage identified by the project inspection plan; repeat when the applicable location, installation condition or controlled source changes. [Enter Project inspection/testing plan and acceptance basis] |
| Responsibility | Proposed: assigned trade supervisor / quality role records the check; review and external sign-off remain with project-assigned persons acting within documented authority. [Enter Assigned performer, reviewer and external control roles] |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Tile cut and penetration coordination record |
| Applicability / remarks | Structural penetrations and service modifications are excluded. Proposed baseline. Confirm applicability and replace with controlled project requirement. |

## TIL-06 · Floor tile bedding, falls and surface geometry

| Inspection / test | Check floor tile installation against the project layout and the specifically adopted substrate, fall, drainage and finish criteria. |
| --- | --- |
| Method | Inspect the floor zone and drainage interfaces using the method and criteria named in project documents; record location-specific observations and readings if required. |
| Acceptance condition | Project source and approved installation details identify the applicable scope, requirements and evidence for this location; record any gap or variance for assigned disposition. [Enter Assigned performer, reviewer and external control roles] [Enter Project inspection/testing plan and acceptance basis] [Enter Project records and handover submission rules] |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | At the work stage identified by the project inspection plan; repeat when the applicable location, installation condition or controlled source changes. [Enter Project inspection/testing plan and acceptance basis] |
| Responsibility | Proposed: assigned trade supervisor / quality role records the check; review and external sign-off remain with project-assigned persons acting within documented authority. [Enter Assigned performer, reviewer and external control roles] |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Floor tiling inspection record |
| Applicability / remarks | No fall, flatness or lippage value is supplied by this baseline. Proposed baseline. Confirm applicability and replace with controlled project requirement. |

## TIL-07 · Wall tile fixing, junctions and alignment

| Inspection / test | Check wall tile rows, corners, junctions, trims, penetrations and interface details as work progresses. |
| --- | --- |
| Method | Compare installation with approved elevations/details, selected system instructions and project appearance criteria; record specific locations. |
| Acceptance condition | Project source and approved installation details identify the applicable scope, requirements and evidence for this location; record any gap or variance for assigned disposition. [Enter Assigned performer, reviewer and external control roles] [Enter Project inspection/testing plan and acceptance basis] [Enter Project records and handover submission rules] |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | At the work stage identified by the project inspection plan; repeat when the applicable location, installation condition or controlled source changes. [Enter Project inspection/testing plan and acceptance basis] |
| Responsibility | Proposed: assigned trade supervisor / quality role records the check; review and external sign-off remain with project-assigned persons acting within documented authority. [Enter Assigned performer, reviewer and external control roles] |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Wall tiling inspection record |
| Applicability / remarks | No coverage ratio or alignment tolerance is inferred. Proposed baseline. Confirm applicability and replace with controlled project requirement. |

## TIL-08 · Grout, movement joints and finish protection

| Inspection / test | Verify grout and designated movement/interface joints are completed to the specified system and other-trade boundary. |
| --- | --- |
| Method | Review joint schedule and product instructions; inspect joint locations, material identity, finish and protection before handover. |
| Acceptance condition | Project source and approved installation details identify the applicable scope, requirements and evidence for this location; record any gap or variance for assigned disposition. [Enter Assigned performer, reviewer and external control roles] [Enter Project inspection/testing plan and acceptance basis] [Enter Project records and handover submission rules] |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | At the work stage identified by the project inspection plan; repeat when the applicable location, installation condition or controlled source changes. [Enter Project inspection/testing plan and acceptance basis] |
| Responsibility | Proposed: assigned trade supervisor / quality role records the check; review and external sign-off remain with project-assigned persons acting within documented authority. [Enter Assigned performer, reviewer and external control roles] |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Jointing and finish protection record |
| Applicability / remarks | Do not substitute sealant or movement-joint design. Proposed baseline. Confirm applicability and replace with controlled project requirement. |

## TIL-09 · Defects, cleaning and tiling handover

| Inspection / test | Close the tiling scope by linking defects/dispositions, cleaning/protection status, product records and the package handover index. |
| --- | --- |
| Method | Reconcile room-by-room records with outstanding items and required client/project closeout documents; leave unresolved items open. |
| Acceptance condition | Project source and approved installation details identify the applicable scope, requirements and evidence for this location; record any gap or variance for assigned disposition. [Enter Assigned performer, reviewer and external control roles] [Enter Project inspection/testing plan and acceptance basis] [Enter Project records and handover submission rules] |
| Reference / locator | Topic guidance only; see Source Register. Enter the actual project document, revision and locator. |
| Trigger / frequency | At the work stage identified by the project inspection plan; repeat when the applicable location, installation condition or controlled source changes. [Enter Project inspection/testing plan and acceptance basis] |
| Responsibility | Proposed: assigned trade supervisor / quality role records the check; review and external sign-off remain with project-assigned persons acting within documented authority. [Enter Assigned performer, reviewer and external control roles] |
| Point / external party | TBC; Proposed: [Enter project-assigned reviewing / release authority] |
| Notice / release | Proposed: Confirm project notice recipient, evidence, timing and release authority; no external control point is assigned. |
| Record | Tiling closeout and handover index |
| Applicability / remarks | Does not assert finished work acceptance or wet-area compliance. Proposed baseline. Confirm applicability and replace with controlled project requirement. |

# Blank matching record prompts

Complete the actual date, observation, result and responsible roles during the work. Blank cells are intentional; this template contains no completed inspection or approval.

## TIL-01 · Scope, room schedule and tile-system release

| Field | Actual entry / evidence |
| --- | --- |
| Room / zone / surface |  |
| Tile product / batch / size ref. |  |
| Drawing / schedule revision |  |
| Open scope decision ref. |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

## TIL-02 · Tile, adhesive, grout and accessory receipt

| Field | Actual entry / evidence |
| --- | --- |
| Tile / adhesive / grout product |  |
| Batch / delivery docket ref. |  |
| Intended room / zone |  |
| Approval / substitution ref. |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

## TIL-03 · Substrate readiness and trade handover

| Field | Actual entry / evidence |
| --- | --- |
| Wall / floor / substrate type |  |
| Room / zone |  |
| Preceding trade release ref. |  |
| Defect / disposition ref. |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

## TIL-04 · Layout, datum and edge/junction set-out

| Field | Actual entry / evidence |
| --- | --- |
| Room / elevation / zone |  |
| Datum / control line ref. |  |
| Approved layout / drawing ref. |  |
| Set-out decision ref. |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

## TIL-05 · Tile cutting, drilling and edge preparation

| Field | Actual entry / evidence |
| --- | --- |
| Tile / room / opening locator |  |
| Coordinated detail ref. |  |
| Service / fixture interface |  |
| Damage / decision ref. |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

## TIL-06 · Floor tile bedding, falls and surface geometry

| Field | Actual entry / evidence |
| --- | --- |
| Floor zone / grid |  |
| Waste / threshold locator |  |
| Geometry / fall source locator |  |
| Observation / photo ref. |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

## TIL-07 · Wall tile fixing, junctions and alignment

| Field | Actual entry / evidence |
| --- | --- |
| Wall / room / elevation |  |
| Course / datum locator |  |
| Corner / floor junction ref. |  |
| Alignment / interface decision ref. |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

## TIL-08 · Grout, movement joints and finish protection

| Field | Actual entry / evidence |
| --- | --- |
| Room / surface |  |
| Joint type / locator |  |
| Grout / sealant product |  |
| Protection / release status ref. |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

## TIL-09 · Defects, cleaning and tiling handover

| Field | Actual entry / evidence |
| --- | --- |
| Room / area |  |
| Defect / disposition ref. |  |
| Product / batch records ref. |  |
| Submission / recipient ref. |  |
| Date / location / lot |  |
| Actual observation / result / unit |  |
| Original evidence / report reference |  |
| Disposition / corrective action / recheck |  |
| Performed by / date |  |
| Reviewed / decision reference |  |

# Topic guidance sources and limitations

## Australian Tile Council, Tiles & Tiling Guide (public contents and topic pages)

Public landing page and contents/topics reviewed: substrate, installation, joints/grouting, lippage, gradients, waterproofing and slip resistance. Full current manual is member access, so its full text was not used.

https://australiantilecouncil.com.au/tiles-and-tiling-guide

Topic taxonomy and distinct checks only; no numerical criteria or full-guide claims.

## training.gov.au, CPCCWF3002 Install floor tiles

Official unit page; planning, work instructions, applicable sources, floor tile installation and assessment context reviewed.

https://training.gov.au/training/details/CPCCWF3002

Stage topics for floor tiling and field records only; unit assessment requirements are not project acceptance criteria.

## training.gov.au, CPCCWF3003 Install wall tiles

Official training.gov.au unit PDF inspected in full (5 pages): application, sequence, wall substrate preparation, cutting, installation, grouting and wall/floor junction scope.

https://training.gov.au/TrainingComponentFiles/CPC/CPCCWF3003_R1.pdf

Topic stages only; no project-specific criteria or licensing conclusions.

## Construction Quality Australia, How to create an Inspection and Test Plan (ITP)

Public industry guide; sections on sequencing activities, method, acceptance source, responsibility, control points and records. Reviewed 2026-10-07.

https://www.cqa.org.au/post/guide-how-to-create-an-inspection-and-test-plan-itp

Structure and record-design guidance only. It does not create project criteria, jurisdictional rules or control points.

Topic guidance is provided for orientation only. Confirm applicable jurisdictional adoption, approved design and contract requirements. No cited source substitutes for the complete controlled project documents or licensed standards.